FRESHWATER INSTRUMENTS

Synthesizer Effect Card

Datasheet

Preliminary

Document FW-DS-102, Revision 1.0, July 2026

About this datasheet Preliminary datasheet for the Effect Card. A few system-level items (final render rate, physical details) are still being settled and are marked TBD. Full operating detail is in FW-UM-102; firmware internals in FW-EN-001.

Contents

Table of contents. In Word, select this line and press F9 to populate it.

# 1. Product Overview

## 1.1 Description

The Effect Card is the capture and mixing hub of the synthesizer system. Two four-channel TLV320ADC6140 converters digitise eight analogue inputs onto a shared TDM bus; the processor de-interleaves, mixes, and processes the channels. The card also supervises the 48 V phantom supply for the microphone input path.

## 1.2 Specifications

| Parameter | Value | Notes |
| --- | --- | --- |
| Processor | STM32H743 (Arm Cortex-M7) |  |
| Audio converters | 2 x TLV320ADC6140 | I2C 0x4C / 0x4D |
| Analogue inputs | 8, simultaneous | TDM, 32-bit |
| Capture rate | 96 kHz | system-level rate TBD (96 or 192 kHz) |
| Mixing and effects | 8-channel mix engine | planned |
| Stereo output | TBD | codec bus reserved on the board |
| Phantom power | 48 V, supervised power-good |  |
| RS-485 console | 115200 baud 8N1, address e: |  |
| Firmware update | USB bootloader | BOOT switch and reset |

# 2. Block Diagram

Figure 1. Capture path.

# 3. Input Channel Map

| Inputs | Converter | I2C address | TDM slots | Selected with |
| --- | --- | --- | --- | --- |
| IN1 to IN4 | TLV320ADC6140 no. 1 | 0x4C | 0 to 3 on data line SD_B | usb ch 1..4 |
| IN5 to IN8 | TLV320ADC6140 no. 2 | 0x4D | 0 to 3 on data line SD_A | usb ch 5..8 |

# 4. Control Reference

## 4.1 Phantom rail truth table

| EN_48V (PC6) | PG_48V (PC7) | Meaning |
| --- | --- | --- |
| Low | Any | Rail disabled (power-up default) |
| High | High | Rail enabled, supply good |
| High | Low | Fault: rail enabled but supply not good. Switch the rail off and investigate. |

## 4.2 Audio domain control

| Signal | MCU pin | Function |
| --- | --- | --- |
| AUDIO_EN | PA7 | Releases both converters from hardware shutdown; asserted at power-up. Converters do not respond on I2C while low. |
| ADC1_INT / ADC2_INT | PC4 / PC5 | Converter interrupt lines |
| INPUT_SW1 / INPUT_SW2 | PB15 / PB14 | Input path switching |

# 5. System Integration

The card runs under the CPU card (Rockchip), which handles voice allocation. RS-485 carries the real-time voice and control traffic; USB carries bulk data such as sample uploads, where its buffering suits large transfers.

| Function | Transport |
| --- | --- |
| Voice / control commands | RS-485 from the CPU card; low-latency, deterministic |
| Sample and bulk data | USB from the CPU card; buffered, suited to large transfers |
| Maintenance console | RS-485 (address e:) and USB CDC, same command set |
| Firmware update | USB bootloader (BOOT switch, reset) |

# 6. Power and Physical

| Parameter | Value |
| --- | --- |
| Supply | TBD |
| Power consumption | TBD |
| Dimensions | TBD |
| Connectors | TBD |
| Mounting | TBD |

# Revision History

| Revision | Date | Changes |
| --- | --- | --- |
| 1.0 | July 2026 | Initial release |