FRESHWATER INSTRUMENTS

Synthesizer Channel Card

Datasheet

Preliminary

Document FW-DS-101, Revision 1.0, July 2026

About this datasheet Preliminary datasheet for the Channel Card. A few system-level items (final render rate, physical details) are still being settled and are marked TBD. Full operating detail is in FW-UM-101; firmware internals in FW-EN-001.

Contents

Table of contents. In Word, select this line and press F9 to populate it.

# 1. Product Overview

## 1.1 Description

The Channel Card generates one synthesizer voice. Digital audio is converted by a CS4304 four-channel DAC and shaped in a true analogue signal path: switched-capacitor filter (SCF), state-variable voltage-controlled filter (VCF), and voltage-controlled amplifier (VCA), with every route selected by electronically controlled switches. Three converter channels provide calibratable control voltages for the analogue blocks.

## 1.2 Specifications

| Parameter | Value | Notes |
| --- | --- | --- |
| Processor | STM32H725 (Arm Cortex-M7) |  |
| Audio converter | CS4304, four channels | 1 audio + 3 CV; I2C 0x60 |
| Audio format | 32-bit, 96 kHz | final render rate TBD (96 or 192 kHz) |
| Analogue processing | SCF, VCF (LP / BP / HP), VCA |  |
| Control voltages | 3, zero-point calibratable | VCA gain, VCF cutoff, VCF resonance |
| SCF cutoff range | 10 Hz to 20 kHz | cutoff = clock / 100 |
| Sample memory | SDRAM, size TBD | for the render engine |
| RS-485 console | 115200 baud 8N1, address c: |  |
| Firmware update | USB bootloader | BOOT switch and reset |

# 2. Block Diagram

Figure 1. Analogue signal routing. Tan blocks are electronically controlled switches.

# 3. Converter Channel Map

| Converter channel | Role | Controlled by |
| --- | --- | --- |
| CH1 | Audio (playback signal) | Host volume, gain 1 |
| CH2 | CV: VCA gain | dc 2 <-100..100> |
| CH3 | CV: VCF cutoff | dc 3 <-100..100> |
| CH4 | CV: VCF resonance | dc 4 <-100..100> |

# 4. Switch Matrix Reference

## 4.1 Switches and active levels

| Switch | MCU pin | Function | Active level |
| --- | --- | --- | --- |
| bypass | PB10 | Dry route: unprocessed signal to output | Low |
| scf | PB14 | SCF output to VCA | Low |
| hp_ctl | PB15 | SCF input select: HP stage (on) or direct (off) | High |
| vcf | PB13 | Signal into the VCF block | Low |
| lp | PB1 | VCF low-pass output to summing point | Low |
| bp | PB0 | VCF band-pass output to summing point | Low |
| hp | PC5 | VCF high-pass output to summing point | Low |
| vca | PB2 | Processed route: VCA output to output | Low |

## 4.2 Output routing truth table

| bypass | vca | Output signal |
| --- | --- | --- |
| off | off | Silence; no path to the output |
| on | off | Dry signal only |
| off | on | Processed signal only |
| on | on | Dry and processed signals, summed |

## 4.3 VCF output selection

| lp | bp | hp | Response delivered to the VCA |
| --- | --- | --- | --- |
| on | off | off | Low-pass |
| off | on | off | Band-pass |
| off | off | on | High-pass |
| on | on | on | Sum of enabled outputs (any combination is permitted) |

## 4.4 SCF input selection

| hp_ctl | SCF input |
| --- | --- |
| on | Through the clocked HP stage |
| off | Direct; the HP stage is bypassed |

## 4.5 SCF clock to cutoff

| Command | Clock | Cutoff |
| --- | --- | --- |
| scf 50 | 50 kHz | 500 Hz |
| scf 100 | 100 kHz | 1 kHz |
| scf 500 | 500 kHz | 5 kHz |
| scf 2000 | 2 MHz | 20 kHz |
| scf 0 | off | SCF inactive |

# 5. System Integration

The card runs under the CPU card (Rockchip), which handles voice allocation. RS-485 carries the real-time voice and control traffic; USB carries bulk data such as sample uploads, where its buffering suits large transfers.

| Function | Transport |
| --- | --- |
| Voice / control commands | RS-485 from the CPU card; low-latency, deterministic |
| Sample and bulk data | USB from the CPU card; buffered, suited to large transfers |
| Maintenance console | RS-485 (address c:) and USB CDC, same command set |
| Firmware update | USB bootloader (BOOT switch, reset) |

# 6. Power and Physical

| Parameter | Value |
| --- | --- |
| Supply | TBD |
| Power consumption | TBD |
| Dimensions | TBD |
| Connectors | TBD |
| Mounting | TBD |

# Revision History

| Revision | Date | Changes |
| --- | --- | --- |
| 1.0 | July 2026 | Initial release |